# SYKON S60 – Casement / Door System

## Technical Catalogue

## 1. System Overview

S60 is a 60 mm aluminium casement system.

The system includes:

- Casement windows (inward and outward opening)

- Hinged doors

- Concealed sash configurations

- Co-planar door configurations

## 2. Performance Summary

| Performance Item | Standard | Result (Achieved) | Status |
| --- | --- | --- | --- |
| Air infiltration | ASTM E283 | Within 2.5 m³/h·m² @ 75 Pa (preliminary test) | Pass |
| Static water penetration | ASTM E331 | 600 Pa (no water ingress) | Pass |
| Wind resistance – serviceability | ASTM E330 | +1400 Pa / +1920 Pa (within L/175) | Pass |
| Wind resistance – safety | ASTM E330 | +2100 Pa / +2880 Pa (no failure) | Pass |

## 3. Additional Certified Performance (EN Standards)

| Performance Item | Standard | Result |
| --- | --- | --- |
| Air permeability | EN 12207 | Class 4 |
| Water tightness | EN 12208 | Class 7A / Class 3A |
| Wind resistance | EN 12210 | Class C3 |

## 4. Measured Performance Data

| Parameter | Value |
| --- | --- |
| Sample type | Hinged door |
| Sample size | 1000 × 2300 mm |
| Glass type | Double glazing |
| Glass thickness | 28 mm |
| Air infiltration | Within permissible limit |
| Water penetration | No leakage at 600 Pa |
| Wind resistance (serviceability) | Up to 1920 Pa |
| Wind resistance (safety) | Up to 2880 Pa |

## 5. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Casement / hinged door system |
| System depth | 60 mm |
| Frame material | Aluminium |
| Thermal construction | Aluminium composite profiles |
| Glazing type | Double glazing |
| Tested glass thickness | 24 mm / 28 mm |
| Sealing system | EPDM gaskets |

## 6. Frame Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| C060-100 | Small frame | 1080 gr/m |
| C060-101 | Medium frame | 1270 gr/m |
| C060-102 | Large frame | 1440 gr/m |
| C060-120 | Storefront frame | 2039 gr/m |
| C060-121 | Entrance door frame | 1880 gr/m |
| C060-190 | Reverse frame | 976 gr/m |
| C060-821 | Storefront frame | 1389 gr/m |

## 7. Sash Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| C060-200 | Inward opening sash (small) | 1331 gr/m |
| C060-201 | Inward opening sash (medium) | 1496 gr/m |
| C060-202 | Inward opening sash (large) | 1830 gr/m |
| C060-211 | Outward opening sash (medium) | 1630 gr/m |
| C060-210 | Outward opening sash (large) | 1846 gr/m |

## 8. Meeting Stile

| Profile | Type | Weight |
| --- | --- | --- |
| C060-500 | Meeting stile | 1270 gr/m |

## 9. Transom / Mullion Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| C060-300 | Small T profile | 1183 gr/m |
| C060-301 | Medium T profile | 1378 gr/m |
| C060-302 | Large T profile | 1545 gr/m |
| C060-303 | Kick-plate (coplanar) | 2247 gr/m |
| C060-310 | Kick-plate (overlap) | 2439 gr/m |
| C060-322 | Mullion T profile | 2712 gr/m |

## 10. Concealed Sash System

| Profile | Type | Weight |
| --- | --- | --- |
| C060-130 | Frame (concealed sash) | 1238 gr/m |
| C060-230 | Concealed sash | 1277 gr/m |
| C060-231 | Slim concealed sash | 717 gr/m |
| C060-330 | T profile (concealed) | 1483 gr/m |

### Glazing Beads (Concealed Sash)

| Profile | Weight |
| --- | --- |
| P060-600 | 551 gr/m |
| P060-601 | 721 gr/m |
| P450-600 | 149 gr/m |

## 11. Co-Planar Door System

| Profile | Type | Weight |
| --- | --- | --- |
| C060-140 | Inward opening frame | — |
| C060-150 | Outward opening frame | — |
| C060-240 | Inward opening sash | 1806 gr/m |
| C060-250 | Outward opening sash | 1816 gr/m |
| C060-540 | Meeting stile | 1070 gr/m |

## 12. Thresholds / Cills

| Profile | Description | Weight |
| --- | --- | --- |
| P060-602 | Threshold | 249 gr/m |
| C060-603 | Threshold | 631 gr/m |
| C060-604 | Threshold | 604 gr/m |
| C060-706 | Kick-plate threshold | 623 gr/m |

### Cills

| Profile | Description | Weight |
| --- | --- | --- |
| P555-101 | Inner cill | 197 gr/m |
| P555-102 | Inner cill | 165 gr/m |
| P555-103 | Inner cill | 267 gr/m |
| P555-104 | Outer cill | 451 gr/m |
| P555-105 | Outer cill | 294 gr/m |
| P555-106 | Outer cill | 113 gr/m |

## 13. Additional Profiles & Accessories

| Profile | Description | Weight |
| --- | --- | --- |
| C555-100 | Groove cap | 38 gr/m |
| P555-109 | Steel reinforcement cover | 1015 gr/m |
| P555-110 | Steel reinforcement cap | 267 gr/m |
| C060-609 | Curtain transaction profile | 865 gr/m |
| P555-111 | Curtain transaction profile | 475 gr/m |
| P555-113 | Swing door adaptor | 518 gr/m |
| P555-114 | Automatic sliding cover | 234 gr/m |
| P555-115 | Automatic sliding cover | 436 gr/m |

## 14. Glazing Range

| Configuration | Thickness |
| --- | --- |
| Double glazing | Supported |
| Tested configurations | 24 mm and 28 mm |

## 15. System Behavior

- Aluminium profile system

- EPDM gasket sealing

- Inward and outward opening configurations

- Multi-configuration system (casement, door, concealed sash, coplanar)

## 16. System Options

- Inward opening

- Outward opening

- Concealed sash

- Co-planar doors

- Storefront and entrance configurations

- Tilt & Turn

- Pull & Slide

- Structural Glazed Door

## 17. Area of Usage / Project Type

Suitable for:

- Residential buildings

- Commercial buildings

- Entrance doors

- Window applications

## 18. Key Technical Highlights

- 60 mm casement system

- Multiple configurations (window, door, concealed, coplanar)

- Tested up to ±2880 Pa structural load

- Water resistance tested up to 600 Pa

- Air permeability within required limits

- Supports double glazing (tested up to 28 mm)

- Maximum glazing for fixed panel and co-planar sash 44mm thk. Maximum glazing for overlap sash panel 54mm thk.