# SYKON SL350 – Sliding System (Non-Thermal)

## Technical Catalogue

## 1. System Overview

SL350 is a non-thermal aluminium sliding system designed for standard residential and commercial applications.

The system supports:

- Sliding doors and windows

- Double and triple track configurations

- Monorail configurations

- Flyscreen integration

It is designed for:

- Internal or semi-external applications

- Standard sliding façade requirements

## 2. Performance Summary

| Performance Item | Status |
| --- | --- |
| Air infiltration / exfiltration | Not specified |
| Water penetration | Not specified |
| Wind resistance | Not specified |

## 3. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Sliding system |
| System concept | Standard sliding |
| Thermal break | None |
| Thermal behavior | Non-thermal system |
| Aluminium alloy | EN AW 6063 |
| Profile thickness | Varies by profile |
| Glazing type | Single and double glazing |
| Sealing system | Brush seals + gaskets |
| Surface finish | Powder coating / anodizing |

## 4. Rail / Frame Profile Options

### Main Rail Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| P350-150 | Double rail | 1204 gr/m |
| P350-151 | Double rail (flyscreen) | 1447 gr/m |
| P350-152 | Triple rail | 1862 gr/m |
| P350-153 | Triple rail (flyscreen) | 2125 gr/m |

### Rail Dimensions (from sections)

| Configuration | Width (mm approx.) |
| --- | --- |
| Double rail | ~86 – 104 mm |
| Triple rail | ~137 – 155 mm |

## 5. Sash Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P350-250 | Sash | 876 gr/m |

## 6. Interlock / Meeting Stile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P350-561 | Interlock 25 mm | 876 gr/m |
| P350-551 | Interlock 30 mm | 1815 gr/m |
| P350-560 | Interlock 25 mm | 716 gr/m |
| P350-550 | Interlock 30 mm | 778 gr/m |

### Meeting Stiles

| Profile | Type | Weight |
| --- | --- | --- |
| P350-400 | Meeting stile | 551 gr/m |
| P350-401 | Corner meeting stile | 831 gr/m |
| P451-452 | Corner sash profile | 613 gr/m |

## 7. Monorail System

| Profile | Type | Weight |
| --- | --- | --- |
| P350-120 | Monorail | 1475 gr/m |
| P350-570 | Monorail post / interlock | 149 gr/m |
| P450-617 / 618 | Monorail cover caps | 459–481 gr/m |
| P450-619 | Flyscreen rail | 524 gr/m |

## 8. Glazing Range (Extracted from Drawings)

| Configuration | Thickness |
| --- | --- |
| Single glazing | 6 mm – 10 mm |
| Double glazing | 22 mm – 28 mm |

### Glazing Options (from gasket combinations)

| Glass Thickness | Wedge Thickness |
| --- | --- |
| 6 mm | 7 mm |
| 7 mm | 6 mm |
| 8 mm | 5 mm |
| 9 mm | 4 mm |
| 10 mm | 3 mm |
| 22 mm | 5 mm |
| 24 mm | 4 mm |
| 26 mm | 3 mm |
| 28 mm | 2 mm |

## 9. Hardware & System Components

- Sliding system compatible with:

  - Master hardware set

  - Roto hardware set

- Bottom rolling mechanism integrated in rail

- Adjustable locking systems

## 10. Gaskets & Sealing Components

| Component | Description |
| --- | --- |
| Brush seals | Sliding sealing |
| Wedge gaskets | Glass fixing |
| Glazing gasket (G1030) | Glass sealing |

## 11. Flyscreen System

| Profile | Description |
| --- | --- |
| P450-620 | Flyscreen sash |
| P450-621 / 625 | Interlock flyscreen |
| P450-606 | Meeting stile |
| P450-626 | Separator / transom |

Supports:

- 25 mm and 30 mm flyscreen configurations

- Integrated sliding flyscreen system

## 12. Drainage & Threshold Options

| Component | Description |
| --- | --- |
| P350-700 | Drainage gutter |
| P450-701 | Drainage grate |

System allows:

- Recessed drainage solutions

- Integrated water evacuation

## 13. Additional Profiles & Accessories

| Profile | Description |
| --- | --- |
| P450-600 | Glazing bead |
| P350-601 | Single glass adaptor |
| P350-600 | Drip profile |
| P555-100 | Drip cap |

## 14. System Behavior

- Non-thermal aluminium construction

- Sliding system with bottom rolling mechanism

- Adjustable glazing through wedge system

- Multi-track configurations for flexibility

- Integrated drainage for water control

## 15. System Options

- Double and triple track systems

- Monorail configuration

- Corner sliding solutions

- Flyscreen integration

- Different interlock widths (25 mm / 30 mm)

## 16. Area of Usage / Project Type

Suitable for:

- Residential projects

- Internal partitions

- Low-rise buildings

- Non-conditioned spaces

- Cost-sensitive developments

## 17. Key Technical Highlights

- Non-thermal sliding system

- Supports single and double glazing

- Multi-track configurations available

- Monorail and flyscreen integration

- Flexible glazing through gasket system

- Cost-efficient and versatile solution