# SYKON SL580 – Thermal Sliding System

## Technical Catalogue

## 1. System Overview

SL580 is a 58 mm thermally broken sliding system / lift & slide system.

The system includes:

- Double rail

- Triple rail

- Hotel type rail

- Sliding configurations with optional flyscreen

## 2. Performance Summary

| Performance Item | Standard | Result (Achieved) | Status |
| --- | --- | --- | --- |
| Air infiltration | ASTM E283 | 1.74 – 2.91 m³/h·m² (600–1000 Pa) | Pass |
| Static water penetration | ASTM E331 | 1000 Pa (no leakage) | Pass |
| Static water penetration (re-test) | ASTM E331 | 1200 Pa (no leakage) | Pass |
| Dynamic water penetration | AAMA 501.1 | 1000 Pa (no leakage) | Pass |
| Wind resistance – serviceability | ASTM E330 | ±1920 Pa (within L/175) | Pass |
| Wind resistance – safety | ASTM E330 | ±2880 Pa (no failure) | Pass |

## 3. Measured Performance Data

| Parameter | Value |
| --- | --- |
| Specimen size | 2.5 m × 3.0 m |
| Area | 7.5 m² |
| Glass type | Double glazing |
| Glass thickness | 30 mm (8 + 16 + 6) |
| Air leakage @ 600 Pa | 1.7386 m³/h·m² |
| Air leakage @ 1000 Pa | 2.912 m³/h·m² |
| Water penetration | No leakage at 1000 Pa and 1200 Pa |
| Dynamic water | No leakage at 1000 Pa |
| Structural load | ±1920 Pa |
| Safety load | ±2880 Pa |

## 4. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Standard Slide / Lift & Slide |
| System designation | 58 mm sliding thermo |
| Thermal break | Present (thermo system) |
| Glazing type | Double glazing |
| Tested glass thickness | 30 mm |
| Sealing system | EPDM gasket |

## 5. Rail / Frame Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| C580-100 | Double rail | 2237 gr/m |
| C580-101 | Double rail with screen | 2610 gr/m |
| C580-102 | Triple rail | 3252 gr/m |
| C580-150 | Hotel type rail | 2083 gr/m |
| C580-120 | Outer part rail | 1516 gr/m |
| C580-121 | Inner part rail | 2149 gr/m |
| C580-122 | Middle part rail | 1456 gr/m |

### Frame Dimensions (from sections)

| Configuration | Width (mm approx.) |
| --- | --- |
| Double rail | ~136.8 mm |
| Double rail with screen | ~169.8 mm |
| Triple rail | ~210.8 mm |

## 6. Sash / Post Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| C580-200 | Glazing sash | 1711 gr/m |
| C580-250 | Slim glazing sash | 1044 gr/m |
| C580-400 | Wide post | 1869 gr/m |
| C580-401 | Narrow post | 1036 gr/m |

## 7. Interlock / Meeting Stile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P580-500 | Interlock | 516 gr/m |
| C580-510 | Meeting stile | 1358 gr/m |

## 8. Glazing Range

| Configuration | Thickness |
| --- | --- |
| Double glazing | Up to 43mm Thk |
| Tested configuration | 30 mm (8 + 16 + 6) |

## 9. Additional Profiles & Accessories

| Profile | Description | Weight |
| --- | --- | --- |
| C580-615 | Glazing bead | 205 gr/m |
| C580-652 | Fixed sash spacer | 383 gr/m |
| C580-700 | Rail cover | 510 gr/m |
| C580-701 | Rail cover | 451 gr/m |
| C580-900 | Anodised rail | 97 gr/m |
| P580-900 | Bottom sealant profile | 545 gr/m |

## 10. Flyscreen System

| Profile | Description | Weight |
| --- | --- | --- |
| P450-604 | Flyscreen sash | 1112 gr/m |
| P450-620 | Flyscreen sash | 635 gr/m |
| P450-607 | Slim flyscreen sash | 689 gr/m |
| P450-621 | Flyscreen interlock | 484 gr/m |
| P450-605 | Flyscreen transom | 749 gr/m |
| P450-606 | Flyscreen meeting stile | 396 gr/m |

## 11. Drainage & Components

| Profile | Description | Weight |
| --- | --- | --- |
| P450-700 | Drainage profile | 1145 gr/m |
| P450-701 | Drainage grill cover | 853 gr/m |

## 12. System Behavior

- Thermally broken aluminium profiles

- EPDM gasket sealing system

- Sliding operation with multi-track configurations

- Lift & Slide Operation is supported with multi-lock system

- Integrated drainage profiles

## 13. System Options

- Double rail

- Triple rail

- Hotel type rail (monorail)

- Flyscreen integration

- Embedded Rails

## 14. Area of Usage / Project Type

Suitable for:

- High-rise buildings

- Coastal and marine environments

- Projects exposed to high wind and water loads

- Residential and commercial façades requiring high performance

## 15. Key Technical Highlights

- 58 mm thermally broken sliding system

- Tested up to ±2880 Pa structural load

- Water tightness up to 1200 Pa

- Air infiltration within 5.4 m³/h·m² criteria

- Supports double glazing (tested at 30 mm)

- Lift & Slide Operation is supported by multi-lock system which gives better acoustic performance