# SYKON SY35 – Slim Curtain Wall System

## Technical Catalogue

## 1. System Overview

SY35 is a 35 mm slim stick curtain wall system designed for conventional façade applications with a reduced sightline profile.

The system provides a slender façade appearance while maintaining structural performance and allows integration of:

- Top-hung windows

- Operable vents

- Integrated façade door systems

It is designed for:

- External façades

- Internal curtain wall applications

## 2. Performance Summary

| Performance Item | Standard | Result | Status |
| --- | --- | --- | --- |
| Air infiltration / exfiltration | ASTM E283 | 600 Pa | Pass |
| Static water penetration | ASTM E331 | 1200 Pa | Pass |
| Wind resistance – serviceability | ASTM E330 | 2000 Pa | Pass |
| Wind resistance – safety | ASTM E330 | 3000 Pa | Pass |

## 3. Measured Performance Data (Representative Sample)

| Parameter | Value |
| --- | --- |
| Air infiltration | Within tested limits up to 600 Pa |
| Water penetration | No water ingress observed |
| Wind resistance | No structural failure under test loads |

## 4. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Stick curtain wall |
| Sightline width | 35 mm |
| Mullion depth range | 60 – 190 mm |
| Aluminium alloy | EN AW 6063 (AlMgSi) |
| Hardness | 12 – 14 HB |
| Profile thickness | 1.75 – 5.0 mm |
| Maximum inertia | Up to Ix = 480.43 cm⁴ |
| Glazing type | Single / Double / Triple |
| Maximum glazing thickness | Up to 50 mm |
| Sealing system | Two-level EPDM gasket |
| Thermal performance (Uf) | 1.3 – 3.3 W/m²K Subject to glass properties and unit dimension |
| Powder coating thickness | Min. 75 microns As specified |

## 5. Mullion / Transom Depth Options

### Mullion Profiles

| Profile | Depth (mm) |
| --- | --- |
| P135060 | 60 |
| P135100 | 100 |
| P135135 | 135 |
| P135142 | 142 |
| P135190 | 190 |
| P135192 | 192 |

### Transom Profiles

| Profile | Depth (mm) |
| --- | --- |
| P135066 | 66 |
| P135106 | 105.5 |
| P135141 | 140.5 |
| P135196 | 195.5 |

## 6. Reinforcement Options

| Profile Range | Reinforcement |
| --- | --- |
| Small depth profiles | Insert tube 85 × 30 mm |
| Medium depth profiles | Insert tube 120 × 30 mm |
| Large depth profiles | Insert tube 175 × 30 mm |
| Alternative | Mullion aluminium sleeves |

## 7. Profile Lengths

| Profile Type | Length |
| --- | --- |
| Mullions / Transoms | 6.0 m standard |

## 8. Glazing Range

### General System Capability

| Configuration | Thickness Range |
| --- | --- |
| Single glazing | Standard |
| Double glazing | Standard |
| Triple glazing | Supported |
| Maximum glazing thickness | Up to 50 mm |

### Monolithic Glass (Capped Detail)

| Configuration | Notes |
| --- | --- |
| Monolithic glazing | 45 mm rebate system |
| Compatible with friction hinge system | Yes |

## 9. Thermal Breaks & Accessories

### Thermal Break Options

| Type | Size (mm) |
| --- | --- |
| T150410 | 10 |
| T150415 | 15 |
| T150420 | 20 |
| T150426 | 26 |
| T150432 | 32 |
| T150438 | 38 |
| T150444 | 44 |

### Glass Supports / Adapters

| Component | Range |
| --- | --- |
| Glass supports | Multiple configurations (20–44 mm range) |
| Adapters | SY-801 to SY-809 series |

### Perimeter Closers

| Type | Size (mm) |
| --- | --- |
| Standard range | 28 / 34 / 40 / 46 / 52 / 58 (NOT BY SYKON) |

## 10. Hardware (Operable Units)

### Friction Hinges

| Arm Length (mm) | Max Height (mm) | Max Width (mm) | Max Weight (kg) |
| --- | --- | --- | --- |
| 261 | 700 | 1200 | 50 |
| 317 | 800 | 1200 | 60 |
| 353 | 1000 | 1300 | 69.5 |
| 414 | 1200 | 1400 | 88.5 |
| 458 | 1400 | 1400 | 102 |
| 511 | 1600 | 1500 | 135 |
| 556 | 1700 | 1500 | 137 |
| 602 | 1800 | 1500 | 139 |
| 707 | 2000 | 1500 | 155 |

### Vent Hardware Components

- Friction stay arms

- Limiter arms

- Cremone handles

- Corner drives

- Strikers

- Adjustable pawls

- Pull-in devices

## 11. System Behavior

- Slim façade design with reduced aluminium exposure

- Designed for efficient water drainage and controlled outflow

- Multi-level sealing ensures:

  - Air tightness

  - Water tightness

  - Acoustic insulation

- Thermal isolators integrated within system

## 12. System Options

- Captured (capped) curtain wall

- Integrated top-hung window systems

- Vent integration within façade grid

- Continuous curtain wall and strip façade options

## 13. Area of Usage / Project Type

Suitable for:

- Villas and residential developments

- Resort and hospitality projects

- Low to mid-rise buildings

- Commercial façades requiring slim profiles

- Architectural façades emphasizing minimal sightlines

## 14. Key Technical Highlights

- Ultra-slim 35 mm sightline

- Reduced aluminium profile width (~40% less than standard systems)

- High structural capacity relative to profile size

- Supports glazing up to 50 mm

- Wide range of mullion depths for structural flexibility

- Integrated operable façade elements

- Optimized thermal and drainage performance