# SYKON SY50 – Curtain Wall System

## Technical Catalogue

## 1. System Overview

SY50 is a 50 mm stick curtain wall system designed for conventional captured glazing as well as 2-side and 4-side structural silicone glazing applications.

The system is suitable for both external and internal façade applications and allows integration of operable elements such as:

- Top-hung windows

- Parallel opening windows

- Tilt & turn windows

- Integrated door systems

## 2. Performance Summary

| Performance Item | Standard | Result | Status |
| --- | --- | --- | --- |
| Air infiltration / exfiltration | ASTM E283 | Tested up to 750 Pa | Pass |
| Static water penetration | ASTM E331 | 1200 Pa | Pass |
| Dynamic water penetration | AAMA 501.1 | 750 Pa | Pass |
| Wind resistance – serviceability | ASTM E330 | 2000 Pa | Pass |
| Wind resistance – safety | ASTM E330 | 4000 Pa | Pass |
| Inter-storey displacement (horizontal) | AAMA 501.4 | ±10 mm | Pass |
| Inter-storey displacement (vertical) | AAMA 501.7 | ±10 mm | Pass |
| Thermal cycling | AAMA 501.5 | Completed cycles | Pass |
| Acoustic performance | EN / ASTM | 38 dB (Rw / STC) | Pass |

## 3. Measured Performance Data (Representative Sample)

| Parameter | Value |
| --- | --- |
| Air infiltration (fixed area @ +300 Pa) | 4.2 m³/h |
| Air infiltration (operable area @ +300 Pa) | 2.7 m³/h |
| Air infiltration after thermal cycling (fixed) | 5.1 m³/h |
| Air infiltration after thermal cycling (operable) | 2.9 m³/h |
| Static water penetration | No water ingress |
| Dynamic water penetration | No water ingress |
| Thermal cycling | 3 full cycles completed |

## 4. Wind Deflection (Representative Values)

### Safety Load – 1.5 × Design Load

| Member | Peak Deflection (mm) | Residual Deflection (mm) |
| --- | --- | --- |
| Mullion | ±11.5 | ±0.3 |
| Transom | ±0.2 | ±0.1 |

### Safety Load – 2.0 × Design Load

| Member | Peak Deflection (mm) | Residual Deflection (mm) |
| --- | --- | --- |
| Mullion | ±16.4 | ±1.6 |
| Transom | ±0.5 | ±0.4 |

## 5. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Stick curtain wall |
| Sightline width | 50 mm |
| Mullion depth range | 50 – 250 mm |
| Aluminium alloy | EN AW 6063 |
| Hardness | 12 – 14 HB |
| Profile thickness | 2.0 – 5.0 mm |
| Maximum inertia | Up to Ix = 1,496.96 cm⁴ |
| Glazing type | Single / Double / Triple |
| Maximum glazing thickness | Up to 50 mm |
| Sealing system | Two-level EPDM gaskets |
| Thermal performance (Uf) | 1.4 – 3.3 W/m²K (subject to glass properties and unit dimension) |
| Powder coating thickness | Min. 75 microns (as specified) |

## 6. Mullion / Transom Depth Options

| Profile | Depth (mm) |
| --- | --- |
| Standard range | 50 |
|  | 70 |
|  | 90 |
|  | 110 |
|  | 130 |
|  | 145 |
|  | 155 |
|  | 175 |
|  | 200 |
|  | 225 |
|  | 250 |

## 7. Corner Mullion Options

| Profile Depth (mm) | Reinforcement |
| --- | --- |
| ~100 mm | 80×40 insert or aluminium sleeve |
| ~157 mm | 140×40 insert or aluminium sleeve |
| ~220 mm | 200×40 insert or aluminium sleeve |
| ~248 mm | 220×40 insert or aluminium sleeve |

## 8. Reinforcement Options

| System Depth Range | Reinforcement |
| --- | --- |
| 50 mm | 40×40 mm insert |
| 70 mm | 60×40 mm insert |
| 90 mm | 80×40 mm insert |
| 110 mm | 100×40 mm insert |
| 130 mm | 120×40 mm insert |
| 145–157 mm | 130×40 / 140×40 mm insert |
| 175 mm | 160×40 mm insert |
| 200–221 mm | 200×40 mm insert |
| 220–250 mm | 220×40 mm insert |
| Alternative | Aluminium sleeves |

## 9. Profile Lengths

| Profile Type | Length |
| --- | --- |
| Standard profiles | 6.0 m |
| Optional | 4.0 m / 6.7 m |

## 10. Glazing Range

### Captured / Capped System

| Configuration | Thickness Range |
| --- | --- |
| Single / Double glazing | 6 mm – 16 mm (typical detail) |
| Double glazing (various setups) | Multiple configurations supported |

### Structural Silicone Glazing

| Configuration | Thickness Range |
| --- | --- |
| Double glazing (SSG) | 24 mm – 50 mm |

### Typical Tested Configuration

- 28 mm DGU (6 + 16 + 6)

## 11. Thermal Breaks & Accessories

| Component | Size / Type |
| --- | --- |
| Thermal breaks | 10 /15 / 20 / 26 / 30 / 36 mm |
| Perimeter closers | 32 / 42 / 44 / 54 mm (not by Sykon) |
| Glazing supports | Multiple configurations |
| Toggle adaptors | Available |

## 12. System Behavior

- Designed with pressure equalization principle

- Integrated drainage system for controlled water evacuation

- Multi-level sealing ensures:

  - Air tightness

  - Water tightness

  - Acoustic performance

## 13. System Options

- Fully capped curtain wall

- 2-side mechanical toggle glazing with silicone weather seal

- 4-side mechanical toggle glazing with silicone weather seal

- Concealed and integrated operable elements

- Compatible with façade-integrated windows and doors

## 14. Area of Usage / Project Type

Suitable for:

- High-rise towers

- Commercial buildings

- Residential developments

- Hospitality projects

- Institutional buildings (schools, offices)

- Mixed-use developments

## 15. Key Technical Highlights

- 50 mm slim sightline system

- Wide structural range (50–250 mm depth)

- High wind resistance up to 4000 Pa

- Water tightness up to 1200 Pa

- Supports glazing up to 50 mm

- Multiple façade configurations (capped & SSG)

- Integrated operable façade elements