# SYKON S50 – Casement System (Non-Thermal)

## Technical Catalogue

## 1. System Overview

S50 is a 50 mm aluminium non-thermal casement system.

The system includes:

- Side-hung windows

- Top-hung windows

- Bottom-hung windows

- Doors (entrance / coplanar / storefront)

## 2. Performance & Certification

| Item | Status |
| --- | --- |
| Thermal insulation | ❌ Non-thermal system |
| Air / Water / Wind testing | ❌ Not available |
| Certification | ❌ Not available |

⚠️ Note: This is a non-thermal system, and no certified performance testing is required

## 3. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Casement |
| System depth | 50 mm |
| Frame material | Aluminium |
| Thermal break | ❌ Not available |
| Opening types | Inward & outward |

## 4. Frame Profile Options

(From profile list – Page 2

S50 Casement Non-Thermal

)

| Profile | Type | Weight |
| --- | --- | --- |
| P 050 100 | Small frame | 832 gr/m |
| P 050 101 | Medium frame | 918 gr/m |
| P 050 102 | Large frame | 1004 gr/m |
| P 050 103 | Medium frame | 1021 gr/m |
| P 050 120 | Storefront frame | 1620 gr/m |
| P 050 121 | Entrance door frame | 1461 gr/m |
| P 050 190 | Reverse frame | 780 gr/m |

## 5. Sash Profile Options

(From page 3

S50 Casement Non-Thermal

)

| Profile | Type | Weight |
| --- | --- | --- |
| P 050 200 | Inward opening – small | 1083 gr/m |
| P 050 201 | Inward opening – medium | 1218 gr/m |
| P 050 202 | Inward opening – large | 1501 gr/m |
| P 050 211 | Outward opening – medium | 1239 gr/m |
| P 050 210 | Outward opening – large | 1846 gr/m |
| P 050 500 | Meeting stile | 980 gr/m |

## 6. Transom / Mullion Profiles

(From page 4

S50 Casement Non-Thermal

)

| Profile | Type | Weight |
| --- | --- | --- |
| P 050 300 | Small ‘T’ profile | 999 gr/m |
| P 050 301 | Medium ‘T’ profile | 1121 gr/m |
| P 050 302 | Large ‘T’ profile | 1207 gr/m |
| P 050 303 | Coplanar door kick-plate | 1571 gr/m |
| P 050 310 | Overlap door kick-plate | 1647 gr/m |
| P 050 322 | Mullion ‘T’ profile | 1963 gr/m |

## 7. Thresholds & Accessories

(From pages 5–6

S50 Casement Non-Thermal

)

| Profile | Description | Weight |
| --- | --- | --- |
| P 050 602 | Threshold | 211 gr/m |
| P 050 603 | Threshold | 472 gr/m |
| P 050 706 | Kick-plate threshold | 502 gr/m |

Additional:

- Inner & outer cills

- Add-on profiles

- Groove caps

- Curtain wall adaptors

## 8. Concealed Sash Option

(From page 7

S50 Casement Non-Thermal

)

| Profile | Description | Weight |
| --- | --- | --- |
| P 050 130 | Frame for concealed sash | 1021 gr/m |
| P 050 230 | Concealed sash | 994 gr/m |
| P 050 330 | ‘T’ profile for concealed sash | 1315 gr/m |
| P 050 600–601 | Glazing beads | 516–678 gr/m |

## 9. Co-Planar Door Option

(From page 8

S50 Casement Non-Thermal

)

| Profile | Description | Weight |
| --- | --- | --- |
| P 050 140 | Co-planar door frame | 996 gr/m |
| P 050 240 | Inward sash | 1204 gr/m |
| P 050 250 | Outward sash | 1196 gr/m |
| P 050 540 | Meeting stile | 813 gr/m |

## 10. Glazing Range

| Configuration | Status |
| --- | --- |
| Single glazing | Supported |
| Double glazing | Supported (profile-dependent) |

## 11. System Configurations

- Standard casement windows

- Overlap doors & windows

- Concealed sash system

- Co-planar doors

- Storefront applications

(Refer to general arrangement drawings – Section C & D in catalogue

S50 Casement Non-Thermal

)

## 12. System Behavior

- Non-thermal aluminium system

- Multi-profile modular structure

- Compatible with multiple opening configurations

- Can integrate with curtain wall adaptors

## 13. Area of Usage / Project Type

Suitable for:

- Residential buildings

- Internal partitions

- Low-rise developments

- Non-thermal façade applications

- Budget-driven projects

## 14. Key Technical Highlights

- 50 mm system depth

- Non-thermal aluminium system

- Wide range of profiles (frames, sashes, transoms, accessories)

- Multiple configurations (casement, concealed, co-planar, storefront)

- No certified performance testing available

- Maximum glazing for fixed panel and co-planar sash 34mm thk. Maximum glazing for overlap sash panel 44mm thk.