# SYKON S77 – Bi - Folding System

## Technical Catalogue

## 1. System Overview

S77 is a 77 mm aluminium bi - folding system.

The system includes:

- Bi - folding door configurations (different variations)

- Integrated hinged door (swing-in or out)

- Multi-panel folding arrangements

- Bottom-supported system with rolling mechanism

## 2. Performance Summary (Tested in Germany – Not Local UAE)

| Performance Item | Standard | Result (Achieved) | Status |
| --- | --- | --- | --- |
| Air permeability | DIN EN 12207 | Class 3 | Pass |
| Water tightness | DIN EN 12208 | Class 6A | Pass |
| Wind resistance | DIN EN 12210 | Class C3 | Pass |
| Resistance to vertical load (sash weight) | DIN EN 13115 | Class 4 | Pass |

## 3. Measured Performance Data

| Parameter | Value |
| --- | --- |
| Test location | Germany (PIV Velbert Laboratory) |
| Test specimen size | 2801 mm × 2500 mm |
| System type | Folding (3-leaf configuration) |
| Opening type | Inward opening |
| Glass type | Double glazing |
| Glass thickness | 30 mm (10 + 14 + 6) |
| Hardware | Roto Patio Fold |
| Sealing system | Multi-gasket system + brush sealing |

## 4. Air Permeability Data (Measured Values)

| Pressure (Pa) | Air Leakage (m³/h·m² approx.) |
| --- | --- |
| 50 Pa | ~2.6 |
| 100 Pa | ~3.8 |
| 150 Pa | ~4.7 |
| 300 Pa | ~8.3 |
| 600 Pa | ~10.8 |

→ Classification: Class 3

## 5. Water Tightness

| Test Pressure | Result |
| --- | --- |
| Up to 250 Pa | No water penetration |

→ Classification: Class 6A

## 6. Wind Resistance

| Parameter | Value |
| --- | --- |
| Wind load classification | Class C3 |
| Max test pressure | ±1200 Pa (based on EN classification) |

## 7. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Bi-folding Doors / Windows |
| System depth | 77 mm |
| Frame material | Aluminium |
| Profile system | Sykotherm Series 75 / 75 plus |
| Glazing type | Double glazing |
| Tested glass thickness | 30 mm |
| Sealing system | EPDM + brush seals |

## 8. Frame Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P077-102 | Low bottom frame | 777 gr/m |
| C077-100 | Frame | 1510 gr/m |
| C077-101 | Frame with wing | 1777 gr/m |
| C077-111 | Regulation frame | 1423 gr/m |
| C077-110 | Corner profile | 3250 gr/m |

## 9. Sash Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| C077-200 | Glazing sash | 1817 gr/m |
| C077-210 | Structural glazing sash | 1790 gr/m |

## 10. Transom / Structural Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| C077-300 | Transom | 1396 gr/m |

## 11. Additional Profiles

| Profile | Description | Weight |
| --- | --- | --- |
| C077-600 | Profile for striker | 1005 gr/m |
| P077-601 | Gasket profile | 205 gr/m |
| P077-602 | Additional wing | 281 gr/m |
| P077-603 | Structural glazing bead | 405 gr/m |
| P060-604 | Anodised glazing bead | 111 gr/m |
| P077-604 | Brush profile | 144 gr/m |

## 12. Glazing Range

| Configuration | Thickness |
| --- | --- |
| Double glazing | Supported |
| Tested configuration | 30 mm |

up to 54mm thick glazing unit

## 13. System Behavior

- Folding panels connected with hinge system

- Bottom-supported rolling mechanism

- Multi-panel opening configuration

- Combination of gasket and brush sealing

## 14. System Options

- Multiple panel configurations

- Inward or Outward opening / folding system

- Structural glazing option

- Standard glazing sash option

- 90 degrees Corner Profile option

## 15. Area of Usage / Project Type

Suitable for:

- Residential villas

- Openable façade systems

- Large opening areas

- Indoor–outdoor transition spaces

- Restaurants and commercial spaces

## 16. Key Technical Highlights

- 77 mm folding system

- Tested in Germany (non-local testing)

- Air permeability: Class 3

- Water tightness: Class 6A

- Wind resistance: Class C3

- Vertical load resistance: Class 4

- Supports double glazing up to 54 mm

Should we mention the limitations for this system??

Panel Height 3.5m

Panel Width 1.2m

Panel Weight 120 kg

This is based on Masters hardware, which we are trying to changed to get a bit of an upgrade on the limitations.