# SYKON SL20 – Minimal Panoramic Sliding System

## Technical Catalogue

## 1. System Overview

SL20 is a minimal panoramic thermally broken sliding system designed to achieve ultra-slim sightlines with maximum transparency.

The system supports:

- Sliding doors and windows

- Double, triple and multi-track configurations

- Large glass panels with minimal aluminium visibility

- Pocket and corner sliding solutions

It is designed for:

- External applications

- Architectural minimal façade concepts

## 2. Performance Summary (tested for project requirement only)

| Performance Item | Standard | Result (Achieved) | Status |
| --- | --- | --- | --- |
| Air infiltration / exfiltration | ASTM E283 | ≤ 5.4 m³/h·m² @ 75 Pa | Pass |
| Static water penetration | ASTM E331 | ~399 Pa (no water ingress) I think this was 180 Pa only when tested?? We need to re-test this | Pass |
| Dynamic water penetration | AAMA 501.1 | No water ingress observed | Pass |
| Wind resistance – serviceability | ASTM E330 | ±1211 Pa (within L/175 limit) | Pass |
| Wind resistance – safety | ASTM E330 | ±1832 Pa (no failure / no damage) | Pass |

## 3. Measured Performance Data (Representative Sample)

| Parameter | Value |
| --- | --- |
| Air infiltration rate | ≤ 5.4 m³/h·m² @ 75 Pa |
| Sample area | ~6.9 m² |
| Total airflow | ~37.3 m³/h |
| Static water penetration | No water ingress |
| Dynamic water penetration | No water ingress |
| Wind deflection limit | L/175 |
| Structural behavior | No failure or permanent deformation |

## 4. Wind Deflection (Representative Values)

### Serviceability Load

| Parameter | Value |
| --- | --- |
| Span reference | ~2730 mm |
| Permissible deflection | ±15.0 mm |
| Measured deflection | Within allowable limits |

### Safety Load

| Parameter | Value |
| --- | --- |
| Residual deflection limit | ≤ ±5.5 mm |
| Measured residual | Within limits |
| Structural integrity | No failure |

## 5. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Minimal sliding system |
| System concept | Slim / panoramic |
| Frame depth (double rail) | ~162 mm |
| Aluminium alloy | EN AW 6063 |
| Hardness | 12 – 14 HB |
| Profile thickness | Varies by profile |
| Thermal break | Polyamide thermal insulation inserts |
| Thermal behavior | Thermally broken system |
| Glazing type | Double / Triple glazing |
| Typical glazing thickness | Variable (up to 45mm thick) |
| Maximum glazing thickness | Up to 45 mm |
| Sealing system | Multi-component gasket + brush system |
| Powder coating thickness | Min. 75 microns |

## 6. Rail / Frame Profile Options

### Main Rail Profiles

| Profile | Type | Weight |
| --- | --- | --- |
| C020-102 | Low bottom frame | 3006 gr/m |
| C020-112 | Standard frame | 2444 gr/m |
| C020-200 | Rail | 918 gr/m |
| C020-201 | Rail | 1056 gr/m |
| P020-500 | Base profile | 1220 gr/m |

### Partial / Add-on Rail Systems

| Profile | Type | Weight |
| --- | --- | --- |
| C020-181 | +1 rail | 1590 gr/m |
| C020-182 | +2 rail | 2755 gr/m |
| C020-191 | +1 jamb | 1295 gr/m |
| C020-192 | +2 jamb | 2450 gr/m |
| P020-010 | Connector | 313 gr/m |

### Assembled Rail Configurations

| Configuration | Width (mm approx.) |
| --- | --- |
| Double rail | ~162 mm |
| Triple rail | ~252 mm |
| Quadruple rail | ~342 mm |

## 7. Sash Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P020-501 | Sash | 1391 gr/m |
| P020-400 | Sash | 1401 gr/m |
| C020-401 | Sash | 1704 gr/m |
| P020-600 | Sash | 837 gr/m |
| P020-300 | Sash | 729 gr/m |
| P020-603 | Sash | 467 gr/m |

## 8. Interlock / Meeting Stile Options

| Profile | Type | Weight |
| --- | --- | --- |
| P020-500 | Interlock | 1220 gr/m |
| P020-501 | Interlock | 1391 gr/m |
| P020-602 | Interlock | 535 gr/m |
| P020-601 | Interlock | 165 gr/m |
| P020-701 | Meeting stile | 1918 gr/m |
| P020-703 | Meeting stile | 1085 gr/m |

### Typical Interlock Dimension

- Interlock width: ~20 mm (ultra-slim design)

## 9. Glazing Range

| Configuration | Thickness |
| --- | --- |
| Double glazing | Supported |
| Triple glazing | Supported |
| Maximum glazing thickness | Up to 45 mm |

## 10. Hardware & System Components

### Sliding Mechanism

- Stainless Steel pulley system

- Pulley block system (alternative configuration)

- Heavy-duty bottom rail sliding system

### Accessories

- Stainless Steel rollers / pulleys

- Stainless Steel Tracks

- Corner connectors

- T-connectors

- Adjustable locking systems

- Sliding guides

## 11. Gaskets & Sealing Components

| Component | Description |
| --- | --- |
| Wedge gaskets | Perimeter sealing |
| Brush seals | Sliding interface sealing |
| EPDM gaskets | Additional sealing |
| Weather pile | Multi-point sealing |

## 12. Drainage & Threshold Options

| Component | Description |
| --- | --- |
| Drainage gutter | Integrated system |
| Drainage grate | Optional |
| Rail drainage channels | Integrated |

System allows:

- Recessed threshold solutions

- Fully Concealed Sill (Floor Tiling on Top)

- Floor-integrated drainage

- Controlled water evacuation

## 13. Additional Profiles & Accessories

| Profile | Description |
| --- | --- |
| Accessory profiles | Various configurations |
| Structural support profiles | Available |
| Steel rail receptor | Included |
| Pulley support profiles | Integrated |

## 14. System Behavior

- Ultra-slim aluminium visibility for panoramic effect

- Thermally broken profiles reduce heat transfer

- Smooth sliding operation using stainless steel pulley system

- Multi-level sealing ensures:

  - Air tightness

  - Water resistance

  - Acoustic improvement

- Integrated drainage ensures efficient water evacuation

## 15. System Options

- Double, triple, and quadruple track systems

- Pocket sliding configurations

- Corner opening systems (90°)

- Locking and non-locking options

- Recessed floor installation

## 16. Area of Usage / Project Type

Suitable for:

- Luxury villas

- High-end residential projects

- Panoramic façades

- Hospitality developments

- Minimal architectural designs

## 17. Key Technical Highlights

- Thermally broken minimal sliding system

- Ultra-slim ~20 mm interlock

- Multi-track configurations up to 4 rails

- Large glass panel capability

- Supports glazing up to 45 mm

- Integrated drainage and recessed threshold options

- Smooth operation with steel pulley system

- Maximum transparency with reduced aluminium profiles