# SYKON SL450 – Standard Sliding / Lift & Slide System

## Technical Catalogue

## 1. System Overview

SL450 Standard Sliding / Lift & Slide is a heavy-duty thermal break sliding system designed for large openings and high-performance applications with an adoptable lift and slide mechanism for better performance ratings.

The system supports:

- Lift & slide doors

- Standard sliding configurations

- Mono, double, triple and multi-track systems

- Large panel configurations with high load capacity

- Embedded sill with gutter drain profile

It is designed for:

- External applications

- High-end residential and commercial façades

## 2. Performance Summary (for standard sliding only)

| Performance Item | Standard | Result | Status |
| --- | --- | --- | --- |
| Air infiltration / exfiltration | ASTM E283 | ±300 Pa | Pass |
| Water penetration | ASTM E331 | ~400 Pa | Pass |
| Wind resistance – serviceability | ASTM E330 | 1920 Pa | Pass |
| Wind resistance – safety | ASTM E330 | 2880 Pa | Pass |

## 3. Measured Performance Data (Representative Sample)

| Parameter | Value |
| --- | --- |
| Air permeability limit | ≤ 5.4 m³/h·m² |
| Sample area | 5.28 m² |
| Total allowable airflow | 28.5 m³/h |
| Water penetration test pressure | ~399 Pa |
| Water spray rate | 3.5 L/m²/min |
| Water test duration | 15 minutes |
| Result | No water ingress |
| Deflection limit | L/175 |
| Structural behavior | No failure or permanent deformation |

## 4. Wind Deflection (Representative Values)

### Serviceability Load

| Parameter | Value |
| --- | --- |
| Design load | ±1920 Pa |
| Span reference | 2250 mm |
| Permissible deflection | ±12.8 mm |
| Result | Within allowable limits |

### Safety Load

| Parameter | Value |
| --- | --- |
| Safety load | ±2880 Pa |
| Residual deflection | Within allowable limits |
| Structural integrity | No damage / no component failure |

## 5. System Basic Technical Data

| Item | Value |
| --- | --- |
| System type | Lift & slide / sliding system |
| System depth | ~106 – 167 mm (depending on rail type) |
| Aluminium alloy | EN AW 6063 |
| Hardness | 12 – 14 HB |
| Profile thickness | Up to ~6.0 mm |
| Thermal break | Polyamide (thermal insulated system) |
| Glazing type | Double glazing |
| Typical glazing thickness | ~32 mm |
| Sealing system | Multi-level sealing system |
| Powder coating thickness | Min. 75 microns |

## 6. Rail / Frame Profile Options

### Standard Rail Systems

| Profile | Type | Width (mm) | Weight |
| --- | --- | --- | --- |
| C450-101 | Double rail | ~106 | 1728 gr/m |
| C450-102 | Triple rail | ~167 | 2626 gr/m |
| C450-103 | Add-on rail | ~75 | 1141 gr/m |
| C450-111 | Double rail | ~86 | 1586 gr/m |
| C450-112 | Triple rail | ~147 | 2484 gr/m |
| C450-113 | Add-on rail | ~65.9 | 1100 gr/m |

### Advanced Rail Systems (450s Series)

| Profile | Type | Weight |
| --- | --- | --- |
| C450-150 | Double rail | 1518 gr/m |
| C450-151 | Triple rail | 1833 gr/m |
| C450-152 | Triple rail | 2398 gr/m |
| C450-153 | Quadruple rail | 2713 gr/m |

### Monorail Options

| Profile | Type | Weight |
| --- | --- | --- |
| C450-120 | Monorail | 1880 gr/m |
| P450-617 / 618 | Cover caps | ~459–481 gr/m |
| P450-619 | Rail | 524 gr/m |

## 7. Sash / Panel Profile Options

| Profile | Type | Weight |
| --- | --- | --- |
| C450-200 | Sash | 1409 gr/m |
| C450-201 | Sash | 1489 gr/m |
| C450-202 | Interlock sash | 841 gr/m |
| C450-203 | Interlock sash | 918 gr/m |
| C450-250 | Sash | 1254 gr/m |
| C450-211 | Slim sash | 1356 gr/m |

## 8. Interlock / Meeting Stile Options

| Profile | Type | Weight |
| --- | --- | --- |
| C450-400 | Meeting stile | 945 gr/m |
| C450-450 | Meeting stile | 765 gr/m |
| P450-500 | Interlock | 383 gr/m |
| P450-501 | Interlock | 335 gr/m |
| P450-550 | Interlock sash | 677 gr/m |
| P450-551 | Reinforced interlock | 1714 gr/m |

## 9. Reinforcement Options

| Profile | Type |
| --- | --- |
| P555-109 | Steel reinforcement profile |
| P555-110 | Steel reinforcement cap |

## 10. Glazing Range

| Configuration | Thickness |
| --- | --- |
| Double glazing | Typical 32 mm |
| Glass build-up | 6 mm + 20 mm gap + 6 mm |

## 11. Additional Profiles & Accessories

| Component | Description |
| --- | --- |
| P450-600 | Glazing bead |
| P450-601 / 611 | Anodized sliders |
| P450-602 / 612 | Rail edge profiles |
| P450-609 / 610 | Architrave and cap |
| P450-623 / 624 | Sliding accessories |

## 12. Flyscreen & Auxiliary Systems

| Profile | Description |
| --- | --- |
| P450-603 / 613 | Flyscreen rails |
| P450-604 / 607 | Flyscreen sash |
| P450-605 | Transom |
| P450-606 | Meeting stile |
| P450-608 | Sash cap |

## 13. Drainage & Threshold Options

| Component | Description |
| --- | --- |
| P450-700 | Drainage gutter |
| P450-710 | Drainage gutter (alternative) |
| P450-701 | Drainage grate |

System allows:

- Recessed floor drainage solutions

- Integrated water evacuation system

## 14. System Behavior

- Lift & slide mechanism enables:

  - Reduced friction during operation

  - Smooth movement of heavy panels

- Designed for large panel sizes and high loads

- Multi-level sealing system ensures:

  - Air tightness

  - Water resistance

  - Thermal insulation

- Integrated drainage system for water management

## 15. System Options

- Lift & slide configuration

- Standard sliding configuration

- Mono, double, triple, and quadruple tracks

- Pocket sliding solutions

- Corner configurations (symmetric corner profiles)

- Flyscreen integration

- Slim sash options

## 16. Area of Usage / Project Type

Suitable for:

- Luxury villas

- High-end residential projects

- Beachfront and large opening façades

- Commercial buildings

- Hospitality and resort developments

## 17. Key Technical Highlights

- Heavy-duty lift & slide system (up to 400kg panel weight)

- Standard Sliding up to 300kg panel weight

- Supports large panel dimensions

- Multi-track configurations up to 4 rails

- Slim interlock options available

- Integrated drainage and threshold solutions

- High structural performance under wind loads

- Smooth operation with advanced sliding mechanism